# Lab Report walkthrough - 4.6.5

# Title: Packet Tracer – Physical Layer Exploration: Connect a Wired and Wireless LAN

## Objectives

The objectives of this lab were:

- To connect network devices using the correct cable types.

- To configure and verify IP addressing based on a predefined addressing table.

- To establish WAN connectivity between routers.

- To integrate wired and wireless LAN networks.

- To verify network connectivity using diagnostic commands.

- To explore and analyze the physical topology view.

## Network Topology Description

The network consists of the following components:

- Cloud (ISP simulation)

- Cable Modem

- Router0 (Core Router)

- Router1 (Branch Router)

- Switch (Wired LAN)

- Wireless Router (Home Gateway)

- Family PC

- Home PC

- Home Printer

- netacad.pka (Server)

- Configuration Terminal

The topology integrates multiple subnetworks connected through routers and WAN links.

IP Addressing Scheme

The following addressing table was implemented: -

| Device | Interface | IP Address |
| --- | --- | --- |
| Router0 | F0/0 | 192.168.2.1/24 |
| Router0 | F0/1 | 10.0.0.1/24 |
| Router0 | S0/0/0 | 172.31.0.1/24 |
| Router1 | S0/0 | 172.31.0.2/24 |
| Router1 | F1/0 | 172.16.0.1/24 |
| Wireless Router | Internet | 192.168.2.2/24 |
| Wireless Router | LAN | 192.168.1.1 |
| Family PC | F0 | 192.168.1.102 |
| Switch | VLAN | 172.16.0.2 |
| Netacad.pka | F0 | 10.0.0.254 |

Each subnet was logically separated to ensure proper routing between networks.

Implementation Procedure

Part 1: Connect to the Cloud

- Router0 F0/0 was connected to Cloud Eth6 using a Copper Straight-Through cable.

- Cloud Coax7 was connected to Cable Modem Port0 using a Coaxial cable.

- Link lights were verified to ensure successful connections.

This simulated ISP connectivity between the internal and external networks.

Part 2: Connect Router0

- Router0 S0/0/0 was connected to Router1 S0/0 using a Serial cable.

- Router0 F0/1 was connected to Netacad.pka using a Cross-Over cable.

- Router0 Console was connected to the Configuration Terminal using a Console cable for management access.

These connections established WAN communication and server access.

Part 3: Connect Remaining Devices

- Router1 F1/0 was connected to the Switch using a Copper Straight-Through cable.

- Cable Modem Port1 was connected to the Wireless Router Internet port.

- Wireless Router Ethernet1 was connected to the Family PC.

- The Home PC and Home Printer were connected wirelessly.

This step integrated both wired and wireless LAN environments.

Part 4: Verification of Network Connectivity

Step 1: Testing Connectivity from Family PC

Command used: ping netacad.pka

Result:

- Successful replies received.

- No packet loss observed.

Web browser test: http://netacad.pka

The webpage opened successfully, confirming HTTP connectivity.

Step 2: Testing Connectivity from Home PC

Command used: ping 172.16.0.2

Result:

- Replies received.

- End-to-end connectivity confirmed.

Step 3: Verifying Router Interfaces

From Configuration Terminal:

- Enable

- show ip interface brief

Output confirmed:

- FastEthernet0/0 – up/up

- FastEthernet0/1 – up/up

- Serial0/0/0 – up/up

All necessary interfaces were operational.

Part 5: Examination of Physical Topology

In the Physical Workspace:

- The Cloud rack contained a connected switch and power distribution equipment.

- The Primary Network showed rack-mounted enterprise devices.

- The Secondary Network displayed dual orange fiber cables representing transmit (Tx) and receive (Rx) paths.

- The Home Network did not include a rack because home environments typically use small, standalone networking devices rather than structured racks.

This comparison demonstrated the structural differences between enterprise and residential network setups.

## Data Flow Analysis

The communication path from the Family PC to the server is as follows:

Family PC
→ Wireless Router
→ Cable Modem
→ Cloud
→ Router0
→ Router1
→ Switch
→ Destination Network

This confirms successful routing between wireless and wired LAN segments.

## Conclusion

This laboratory exercise successfully demonstrated the integration of wired and wireless LAN networks within a routed environment.

The following were achieved:

- Correct selection and use of network cables.

- Proper IP addressing configuration.

- Successful WAN routing between routers.

- Verified end-to-end connectivity.

- Clear understanding of logical and physical network topologies.

All lab objectives were met, and the network operated as expected.