# Lab Walkthrough- 4.7.2

# Packet Tracer – Connect the Physical Layer

## Objectives

The objectives of this lab were to: -

- Identify the physical characteristics of internetworking devices.

- Examine the management, LAN, and WAN interfaces of a Cisco router.

- Select and install appropriate expansion modules for connectivity.

- Connect devices using the correct cable types.

- Verify network connectivity between wired and wireless devices.

## Part 1: Identify Physical Characteristics

### Step 1: Identify Management Ports of a Cisco router.

The Cisco 1941 East router was examined in the Physical tab.

Observed Management Ports:

- Console Port (RJ-45)

- AUX Port

The console port is used for local configuration through a console cable, while the AUX port supports remote access via modem.

### Step 2: Identify LAN and WAN Interfaces

Using the CLI command: show ip interface brief

The following interfaces were identified:

LAN Interfaces:

- GigabitEthernet0/0

- GigabitEthernet0/1

WAN Interface:

- Serial0/0/0

The VLAN1 interface was also listed, but it is a virtual interface that exists only in software.

### Step 3: Check Default Bandwidth

Commands used:

show interface gigabitethernet 0/0
show interface serial 0/0/0

### Observed Default Bandwidth:

- GigabitEthernet → 1,000,000 Kbit (1 Gbps)

- Serial Interface → 1544 Kbit (1.544 Mbps)

The serial bandwidth value is used by routing protocols and does not represent actual physical speed.

### Step 4: Identify Expansion Slots

The Cisco 1941 router contains:

- Two HWIC expansion slots for additional modules.

The 2960 switch does not support modular expansion.

## Part 2: Select the correct modules for connectivity

To connect PCs directly to the router without purchasing a new switch, the following module was selected:

Module Installed on East Router:

- HWIC-4ESW (4-Port Ethernet Switch Module)

This module provides four additional FastEthernet ports, allowing up to four hosts to connect directly to the router.

For fiber connectivity between switches:

Module Installed on Switch2:

- Gigabit SFP Fiber Module

Devices were powered off before inserting modules and powered on after installation.

## Part 3: Connect Devices

Devices were connected according to the provided connection table using the correct cable types.

Cable Types Used:

- Copper Straight-Through:

  - PC ↔ Switch

  - Router ↔ Switch

  - Router ↔ PC (via HWIC module)

- Copper Cross-Over:

  - Switch ↔ Switch (Switch4 to Switch3)

- Fiber:

  - Switch3 ↔ Switch2

- Serial DCE:

  - East ↔ West (Serial0/0/0)

After completing all connections, the Packet Tracer score increased, confirming correct cabling.

## Part 4: Check Connectivity

Step 1: Verify Interface Status

Command used: show ip interface brief

All required interfaces displayed:

Status: up
Protocol: up

This confirmed correct physical and logical connectivity.

### Step 2: Test PC Connectivity

Initially, PC1 showed:

IP Address: 0.0.0.0

This indicated that DHCP was not assigned.

After enabling DHCP on the PC, it received a valid IP address.

Ping test: ping 172.29.1.1

Result:

Reply from 172.29.1.1
Packets: Sent = 4, Received = 4, Lost = 0

Connectivity was successful.

### Step 3: Wireless Testing

Laptop:

- Wireless0 interface enabled

- Successfully accessed www.cisco.srv

Tablet:

- Wireless0 tested successfully

- WIFI disabled

- 3G/4G Cell1 enabled

- Successfully accessed web page via cellular connection

### Step 4: Final Connectivity Verification

All PCs were able to:

- Ping their default gateway

- Access the web server

- Communicate across networks

This confirmed full network functionality.

## Conclusion

This lab successfully demonstrated:

- Identification of router physical components

- Installation of correct expansion modules

- Proper cable selection and physical connections

- Interface verification using CLI commands

- DHCP troubleshooting and IP assignment

- Wired and wireless connectivity testing

All required tasks were completed successfully, and full connectivity was achieved across the network.