# Lab Walkthrough – 4 (2.9.1)

# Title: Basic Switch Configuration and End-to-End Connectivity

## Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Class-A | VLAN 1 | 128.107.20.10 | 255.255.255.0 |
| Class-B | VLAN 1 | 128.107.20.15 | 255.255.255.0 |
| Student-1 | NIC | 128.107.20.25 | 255.255.255.0 |
| Student-2 | NIC | 128.107.20.30 | 255.255.255.0 |

## Objective

The objective of this lab was to:

- To configure hostnames and IP addresses on two Cisco IOS switches using the Command Line Interface (CLI).

- To use Cisco IOS commands to control and secure access to device configurations.

- To save the running configuration to NVRAM to ensure settings are retained after reboot.

- To configure two host devices with appropriate static IP addresses.

- To verify end-to-end connectivity between the two PC end devices using ping commands.

## Part 1: Set Up the Network Topology

### Step 1: Device Placement

The following devices were placed in Cisco Packet Tracer according to the given topology:

- Two Cisco Switches: Class-A and Class-B

- Two PCs: Student-1 and Student-2

The switches were connected, and each PC was connected to its respective switch.

### Step 2: Cable Connections

The devices were connected as follows: -

- Class-A FastEthernet0/1 → Class-B FastEthernet0/1 (Copper Crossover Cable)

- Student-1 → Class-A FastEthernet port (Copper Straight-Through Cable)

- Student-2 → Class-B FastEthernet port (Copper Straight-Through Cable)

### Console Connection: -

- A Console cable was connected between the PC (RS-232 port) and the Switch Console port to access the switch CLI for configuration.

- Terminal was opened from the PC to configure the switch using Cisco IOS commands.

### After connecting: -

- All devices were powered on.

- Link indicators (green triangles) confirmed active physical connections.

- The console session successfully displayed the switch CLI prompt, confirming proper console access.

This matches the provided network diagram and allowed successful configuration and verification of the network devices.

## Part 2:  Switch Configuration

### In Part 2, both switches were configured using console access.

Step 1: Access Switch via Console

- Connected the console cable between the PC and Class-A

- Opened Terminal

- Entered privileged mode using: enable

Each switch was accessed using a console connection via the CLI.

### Step 2: Configuration of Class-A

Entered global configuration mode: configure terminal

The following configurations were applied:

- Hostname set to Class-A

- Enable secret password configured as: enable secret 6EBUp

- Console password configured as:

line console 0
password xAw6k
login
exit

- VTY password configured as:

line vty 0 15
password xAw6k
login
exit

- VLAN 1 interface assigned IP address:

interface vlan 1
ip address 128.107.20.10 255.255.255.0
no shutdown
exit

- Password encryption enabled: service password-encryption

- MOTD banner configured: banner motd # Unauthorized access is prohibited #

- Save Configuration:

end
copy running-config startup-config

### Step 3: Configure Class-B

The same steps were repeated for Class-B with the following changes:

- Hostname set to Class-B

- VLAN 1 IP address configured as: 128.107.20.15 255.255.255.0

- Configuration was saved to NVRAM.

password xAw6k

login
             exit

- Password encryption: service password-encryption

- MOTD banner: banner motd # Unauthorized access is prohibited #

### Step 4: Saving Configuration

To ensure the configuration was not lost after reboot, the following command was used:

copy running-config startup-config

Configuration was successfully saved on both switches.

## Part 3: PC Hosts Configuration

Both PCs were configured manually using static IP addressing: -

### Student-1 Configuration:

- IP Address: 128.107.20.25

- Subnet Mask: 255.255.255.0

- Default Gateway: Not required for this lab

Student-2 Configuration:

- IP Address: 128.107.20.30

- Subnet Mask: 255.255.255.0

- Default Gateway: Not required for this lab

Both PCs were placed in the same /24 network.

## Part 4:  Connectivity Testing

Connectivity was verified using the ping command.

From Student-1 Command Prompt: ping 128.107.20.30

Successful replies were received, confirming:

- Proper switch configuration

- Correct IP addressing

- Active physical connections

- End-to-end Layer 2 connectivity

## Conclusion

This lab provided a practical understanding of:

- Physical topology configuration

- Static IP addressing

- Basic switch security configuration

- VLAN interface configuration

- Password encryption

- Connectivity verification

- Saving configuration to NVRAM

The lab was completed successfully. This demonstrates the correct implementation of basic switch configuration and LAN connectivity using Cisco IOS.